Autonomous Power Management for Interlinked AC-DC Microgrids

Inam Ullah Nutkani, Senior Member, IEEE, Lasantha Meegahapola, Senior Member, IEEE, Loh Poh Chiang Andrew, Senior Member, IEEE, and Frede Blaabjerg, Fellow, IEEE

Abstract—The existing power management schemes for inter- linked AC-DC microgrids have several operational drawbacks. Some of the existing control schemes are designed with the main objective of sharing power among the interlinked microgrids based on their loading conditions, while other schemes regulate the voltage of the interlinked microgrids  without  considering the speciﬁc loading conditions. However, the existing schemes cannot achieve both objectives efﬁciently. To address these issues, an autonomous power management scheme is proposed, which explicitly considers the speciﬁc loading condition of the DC micro- grid before importing power from the interlinked AC microgrid. This strategy enables voltage regulation in the DC microgrid, and also reduces the number of converters in operation. The proposed scheme is fully autonomous while it retains the plug-n- play features for generators and tie-converters. The performance of the proposed control scheme has been validated under different operating scenarios. The results demonstrate the effectiveness of the proposed scheme in managing the power deﬁcit in the DC microgrid efﬁciently and autonomously while maintaining the better voltage regulation in the DC microgrid.

Index Terms—Autonomous control, distributed control, droop control, hybrid microgrids, interlinked microgrids, power management.

T

I. INTRODUCTION

HE technical advancement in power electronics is playing an important role in the deployment of renewables and alternative energy technologies [1]–[3] which have so far been widely realized in different forms of network topolo- gies and conﬁgurations [4], [5]. Similarly, they have been controlled and managed using various control strategies and architectures [6], [7]. Their network topologies and control strategies are mainly determined to maximize the beneﬁts while meeting the load requirements. At present, renewable and alternative energy technologies are widely deployed in microgrids. The deployment of these new technologies in the form of a microgrid is preferred due to several advantages,

Manuscript received October 20, 2016; revised April 13, 2017; accepted

May 31, 2017. Date of publication March 30, 2018; date of current version

December 27, 2017.

- U. Nutkani (corresponding author, e-mail: inam.nutkani@rmit.edu.au), and L. Meegahapola are with the School of Engineering, Royal Melbourne Institute of Technology (RMIT), Melbourne 3000, Australia.

L. P. Chiang A. is with the Department of Electronics Engineering, The Chinese University of Hong Kong, Shatin, N.T., Hong Kong, China.

F. Blaabjerg is with the Department of Energy Technology, Aalborg University, Aalborg, Denmark.

DOI: 10.17775/CSEEJPES.2016.01510

such as optimal utilization of resources, improved power qual- ity and enhanced supply reliability [8]–[10]. Recently, more advanced grid architectures have emerged including the zone- based grid architectures [11], multi-microgrids [12]–[15], in- terlinked AC-AC microgrids [16], [17] and interlinked AC-DC microgrids [18]–[22]. The main objective of these advanced network architectures is to exploit maximum beneﬁts from renewables and alternative energy resources. For example, by interconnecting two or more microgrids, it will enable reserve sharing, support voltage and frequency, and ultimately enhance the overall reliability and resilience of interlinked microgrids. The interlinking arrangement between two or more micro- grids or with utility grids primarily depends on the overall objectives, as well as the control and management scheme used in individual microgrids. The microgrids can be interlinked di- rectly or through harmonizing tie-converters. The harmonizing tie-converters are primarily used when two or more microgrids have different operating voltages and/or frequencies. The tie- converters are also essential if the microgrids to be interlinked have different control strategies and the power ﬂow among them needs to be regulated [16]. Similarly, the interlinking of the DC microgrid with the utility grid or another AC microgrid also requires tie-converters to regulate the power ﬂow among other functionalities, and that has been investigated under vari- ous scenarios in the published literature [18]–[22]. In [18], the demand-droop control has been proposed for the interlinking or tie-converters of the AC-DC microgrids. The power ﬂow action is determined based on the normalized terminal voltage and frequency of the droop controlled interlinked AC-DC microgrids. This scheme enables autonomous power transfer between two interlinked microgrids based on their relative loading condition. The power ﬂow decision based on the relative loading may cause the interlinking converter to operate continuously, and thus it may result in unnecessary operational losses. The same power  sharing  scheme  has been extended to interlinked microgrids with a storage system [19]. This scheme is further improved with the progressive auto-tuning to minimize the energy ﬂow through interlinking converters [20]. The proposed auto-tuning enables the power transfer only when one microgrid is heavily-loaded, and another microgrid is lightly-loaded. The droop based power sharing concept has been further investigated for  different operating conditions of the interlinked AC and  DC  microgrids  in  [21]. In [22], the power management strategy is presented for  a  three- port system comprising AC, DC and a storage network. The decision about the power sharing is based on the loading con-

2096-0042 © 2016  CSEE

dition of the interlinked networks which is principally the same as presented in [20]. In addition, a communication based multilevel supervisory control is proposed to reduce the oper- ation of interlinking converters. Another power management scheme presented in [23] for the interlinked AC-DC microgrid has an objective to regulate the voltage of the DC microgrid without taking into consideration the speciﬁc loading level of the generators. This scheme can be implemented only at a single tie-converter, hence limits the plug-n-play feature. In addition, a few centralized power management schemes have been investigated for interlinked AC-DC microgrids [24], [25]. The key concern with the centralized schemes is the reliability associated with the fast communication links. Therefore, the decentralized schemes are usually preferred.

So far the published decentralized power sharing schemes for interlinked AC-DC microgrids are either entirely based on droop principle or voltage regulation. The droop based power sharing schemes transfer power based on relative loading of the interlinked microgrids. The power transfer during a con- tingency or uneven loading condition supports the voltage and

simultaneously regardless of the power transfer demand, the subsequent tie-converter only activates once the ﬁrst converter power capacity has been saturated. The proposed scheme is fully autonomous with enhanced features.

- CONTROL OF AC AND DC MICROGRIDS

The considered DC microgrid includes a non-dispatchable generator (solar-PV) and dispatchable generators (microtur- bine, fuel-cell) and loads, as shown in Fig. 1. The non- dispatchable-solar PV system is set to operate in current con- trol mode and thus extracts maximum power at all the times. The dispatchable generators are typically used for ﬁrming the renewable capacity and can be controlled either through a centralized or decentralized control scheme. The decentralized droop scheme is the most widely used and preferred, as it       is simple and reliable. Therefore, the traditional droop (P-V) scheme has been used for the dispatchable generators of the DC microgrid (see Fig. 1), which is given by

Vdc,ref,i = Vdc,max − ∂dc,iPdc,i

frequency but does not regulate the voltage and/or frequency of

∂	= Vdc,max − Vdc,min = ΔVdc

(1)

the interconnected microgrids. However, these schemes enable

dc,i

Pdc,max,i

Pdc,max,i

plug-n-play feature for the interlinking converters. With this feature, in case there is more than one interlinking converter,

where, i is the DC generator number (i = 1, 2, 3... ); Vdc,ref,i

is the reference voltage of ith generator; Pdc,i is the output

all  converters  will  operate  regardless  of  the  overall power

transfer  requirement.  This  may  incur  unnecessary converter

power of ith generator; Vdc,max and (V

dc,min

= Vdc,nom,TC1 )

operational losses. Contrarily, the voltage regulation schemes regulate the voltage of the DC microgrid without considering speciﬁc loading conditions of the generators, and lacks the plug-n-play feature for tie-converters. These shortcomings can be speciﬁcally addressed using the proposed control scheme  in this paper.

The proposed autonomous power management scheme for interlinked AC-DC microgrids takes into consideration the speciﬁc loading condition of the generators, and transfers power from AC to DC microgrid during its peak-load demand, and also regulates the voltage of the DC microgrid. The proposed scheme enables the plug-n-play feature for tie-

are the deﬁned maximum and minimum voltage; Pdc,max,i is the maximum or rated power of ith generator; and ∂dc,i is the droop gain of ith generator.

Based on (1), the voltage reference for the droop controlled generators 1 and 2 can be calculated by (2) and (3). As generators 1 and 2 share common DC bus voltage (i.e.,

Vdc,ref,1 = Vdc,ref,2), (2) and (3) can be equated and rewritten

by (4), which demonstrates that the droop controlled generator

will share proportional power according to their rated power capacity.

Vdc,ref,1 = Vdc,max − ∂dc,1Pdc,1	(2)

Vdc,ref,2 = Vdc,max − ∂dc,2Pdc,2	(3)

converters and reduces the number of converters in operation to avoid unnecessary losses. In the considered scenario, the DC microgrid has inadequate generation capacity due to the high variability of the loads and renewable generation. The

Pdc,1

→ P

∂dc,1Pdc,1 = ∂dc,2Pdc,2

dc,max,1

Pdc,2

=

Pdc,max,2

Pdc,i

=

Pdc,max,i

(4)

AC microgrid is considered to have regulated voltage and frequency as well as the surplus power to transfer to the DC microgrid during its peak demand or contingency condition. To achieve the features discussed above, a hybrid droop and voltage regulation mode control has been proposed for the tie-converters in interlinked AC-DC microgrids. The proposed control scheme relies on the tie-converter terminal voltage information to determine the overall  loading  condition  of  the droop-controlled DC microgrid. Based on the set loading threshold, the tie-converter starts automatically and transfers power to the DC microgrid during the peak-load demand or contingency condition in the DC microgrid. With the proposed hybrid control mode, the voltage of the DC microgrid is regulated at a deﬁned nominal level. In addition, the proposed scheme allows interfacing more than one tie-converters, but as opposed to the existing scheme where all tie-converters operate

The equality in (4) is based on the fact that the voltage at

the generator terminals is the same. Practically, the voltage at all the generator terminals is not the same due to the fact that they are connected through feeders/cables of different lengths. This voltage mismatch at the generator terminals affects the power sharing accuracy, which needs to be compensated by using any of the appropriate compensation methods [26], [27]. The droop equation with compensation of the feeder voltage drop can be rewritten by

Vdc,ref,i  = Vdc,max − ∂dc,iPdc,i + idc,iXi.	(5) The voltage of the droop controlled DC microgrid will vary

with the changing load, but within the deﬁned permissible range. For the considered DC microgrid, the voltage range with increased aggregated loading is shown in Fig. 1 (bottom- left). For the droop controlled generators, the voltage range is

AC Grid (AC-G)

Characterized by regulated Frequency and Voltage

Pac,aggregate Qac,aggregate

Utility grid

AC generator

Solar PV

Load

Tie Converters (TCs)

•With the proposed hybrid control (droop and voltage control mode), tie-converters autonomously transfer power from AC to DC microgrid when the load demand in DC microgrid is high and all generators are heavily loaded (setable threshold).

•At the same time, tie-converters autono- mously regulate the voltage of DC micro- grid to avoid further degradation.

•The number of converters in operation are reduced to avoid unnecessary operati- onal losses.

•The proposed scheme is fully autonomous and allow plug-n-play of generators and tie-converters.

50 Hz

415 V

Power

Frequency AC voltage

Pac,G

415 V AC, 50 Hz

Pac,1

1

Pac,TC

Pac,2

2

Pac,TC

Pac,Load

Pdc,max,TCx

(100-H)%×Pdc,max,TCx

1

%L×Pdc,max,TCx

Droop 2 control mode

x=2 x=1

Voltage control mode

Droop 1 control mode

397.5 V

400.0 V

402.5 V

DC voltage

Tie converter 1

400 V DC (±5%)

Pdc,TC

Tie

converter 2

2

Pdc,TC

DC Grid (DC-G)

•Dispatchable generators are droop controlled.

•DC grid voltage is regulated within defined range.

•Non-dispatchable generators produce maximum power.

Pdc,aggregate

Pdc,1

Pdc,2

Pdc,3

Pdc,Load

Vdc,nom,TC =397.5 V

Vdc,nom,TC =400.0 V

Vdc,start,TC =402.5 V

Vdc,max=420 V

DC voltage

2

1

1

Droop control

Droop control

Fuel cell

Load profile

Irradiance profile

Load (Ppeak: 30 kW)

Microturbine (Gen 1: 10 kW)

(Gen 2: 5 kW)	Solar PV

(Gen 3: 10 kW)

Fig. 1. Interlinked AC-DC microgrids and their control strategy.

≤

set between 395 V and 420 V, indicating that the generators will deliver no-power at 420 V and 100% power at 395 V. Once the DC generators are heavily loaded (e.g., 402.5 V at 80% generators loading), the tie-converters will start to import power from the AC microgrid to meet the peak load demand, and also regulate the voltage of the DC microgrid.

For the example of interlinked microgrids in Fig. 1, the volt- age and frequency of the AC microgrid is considered stiff. The AC microgrid can be droop controlled with secondary voltage and frequency regulation, or operating in grid-connected mode. The characteristics of the AC microgrid are shown in Fig. 1, where the voltage and frequency are constant at nominal value (e.g., 50 Hz and 415 V). In addition, the AC microgrid has sufﬁcient generation capacity to meet its local demand and export surplus power to the DC microgrid which has been demonstrated through the proposed autonomous  control  of the tie-converters. The details of the tie-converters control are given in Section III.

- PROPOSED HYBRID CONTROL OF TIE-CONVERTERS The power rating of dispatchable generators or storage

systems for ﬁrming the renewable capacity depends on the variability of the renewable source and loads in the micro- grid. The high variability of renewables and loads requires dispatchable generators or storage systems with a high power rating, which may or may not be a viable solution. Alter- natively, the microgrid with inadequate generation capacity could be interconnected with another microgrid or utility grid,

directly or through harmonizing converters. The tying of a  DC microgrid with a AC microgrid or utility grid is only possible through tie-converters, as shown in Fig. 1. In the proposed interlinked system, the AC microgrid is characterized as a regulated voltage and frequency system with adequate generation capacity, whereas the DC microgrid is characterized as a droop controlled system with inadequate generation capacity due to the high variability of the renewable and loads. During the peak demand or the low renewable power output, the power deﬁcit in the DC microgrid is managed by importing power from the AC microgrid. Ideally, it can be achieved efﬁciently and autonomously with the proposed control of the tie-converters.

In summary, the control scheme of the tie-converters is developed based on the following objectives: 1) To transfer power from the AC to DC microgrid during the peak load demand or generation contingency in the DC microgrid; 2) To minimize the power transfer losses, e.g., tie-converter should operate only during the peak-load demand in the DC micro- grid, and the number of tie-converters in operation should be based on power transfer demand; 3) To  regulate the voltage  of the droop controlled DC microgrid; 4) To achieve fully autonomous control without depending on the communication network; 5) To enable the plug-n-play feature for tie-converters and generators.

Unlike the existing schemes for the interlinked AC-DC microgrids [18]–[22], a hybrid droop and voltage regulation mode control is proposed for the tie-converters and the mathematical form of the proposed control scheme is given by:

14	CSEE JOURNAL OF POWER AND ENERGY SYSTEMS, VOL. 4, NO. 1, MARCH 2018

Vdc,ref,TCx =

⎧⎪⎨

Off;	Vdc > Vdc,start,TCx

Vdc,start,TCx − δL,TCx × Pdc,TCx;	0 ≤ Pdc,TCx ≤ L% × Pdc,max,TCx

⎪⎩

Vdc,nom,TCx;	L% × Pdc,max,TCx < Pdc,TCx < (100−H)%×Pdc,max,TCx

Vdc,nom,TCx−δH,TCx[Pdc,TCx−(100−H)% × Pdc,max,TCx]; (100 − H)% × Pdc,max,TCx ≤ Pdc,TCx ≤ Pdc,max,TCx

where TCx represents the tie-converter number (x = 1, 2, 3..);

Vdc is the DC microgrid voltage; Vdc,ref,TCx  is the  reference

voltage of xth tie-converter; Vdc,start,TCx is the threshold voltage to start of xth tie-converter; Vdc,nom,TCx is the nom- inal  voltage  to  be  regulated  by  xth  tie-converter;  Pdc,TCx is the DC power output of xth tie-converter; Pdc,max,TCx

is the maximum power limit of xth tie-converter;  L% and H% are the percentage of tie-converer rated power allocated for droop1 and 2 mode, respectively; Vdc,nom,TCx+1 is the

−	×

−	×

mum power; δL,TCx = (Vdc,start,TCx Vdc,nom,TCx)/(L% Pdc,max,TCx) is the droop 1 gain (at low power) of xth tie- DC microgrid voltage when xth tie-converter transfers maxi- converter; δH,TCx = (Vdc,nom,TCx Vdc,nom,TCx+1)/(H% Pdc,max,TCx) is the droop 2 gain (at high power) of xth tie- converter.

As shown in Fig. 1, tie-converter 1 starts in droop 1 control

(6)

voltage of the DC microgrid is regulated at the nominal value, which is not the case with the existing power management schemes for interlinked microgrids. The performance of the proposed scheme has been validated for different load operat- ing scenarios, as described in Section IV.

- PERFORMANCE VALIDATION

The performance of the proposed scheme has been validated for two different scenarios of the DC microgrid. In the ﬁrst scenario, the microgrid comprises a dispatchable microturbine (Gen 1), fuel cell (Gen 2) and variable load. In the second scenario, a non-dispatchable solar PV generator (Gen 3) is added to scenario 1. The system parameters are summarized  in Tables I–III.

TABLE I

CONTROL MODE OF DC AND AC MICROGRIDS

mode  when  the  voltage  in  the  DC  microgrid  drops  to the

set threshold of Vdc,start,TCx. This voltage threshold implies that all the generators in the DC microgrid are heavily-loaded (e.g. over 80% loaded).  The  start  of  the  tie-converter  in  the droop control mode enables a smooth transition to the voltage regulation mode at the set condition i.e., Pdc,TCx >

Entity	Control Mode

Islanded-microgrid with regulated AC microgrid	voltage and frequency

Grid-connected mode

Tie-converter	Hybrid droop and voltage control  mode

Dispatchable generators	Droop controlled

DC microgrid

×

L%	Pdc,max,TCx. During the voltage regulation mode, the

tie-converter imports power from the AC microgrid to meet

the DC microgrid peak power demand as well as regulate its

Non-dispatchable

generators

TABLE II

Current control

mode with MPPT

voltage to be set to the nominal value of Vdc,nom,TCx.

DC MICROGRID PARAMETERS

Furthermore,   unlike   the   parallel   operation   of   all  tie-

converters in the existing schemes, the converters operation has been prioritized. The ﬁrst tie-converter only starts when

Description	Parameter	Value

Voltage	Vdc (V)	400 (+5%, –1.25%)

all the generators in the DC microgrid are heavily-loaded. Once the ﬁrst tie-converter power capacity is near to saturation

Micro-turbine	Pdc,max,1 (kW)

(V/kW)

∂dc,1

Solar PV	Pdc,max,3 (kW)	10

Fuel cell	Pdc,max,2 (kW)

10

2.5

5

dc,TCx

at P

= (100 − H)% × P

, its control mode

∂dc,2 (V/kW)	5

dc,max,TCx

is changed from the voltage regulation to droop 2 control mode to allow minor voltage drop. This minor voltage drop caused by the droop 2 control mode will enable the next tie-converter to start its operation. In case of failure of the

Load	PLoad,peak (kW)	25

TABLE III

AC MICROGRID AND TIE CONVERTER PARAMETERS

ﬁrst tie-converter, the second tie-converter will automatically

start its operation followed by the voltage drop due to high  load demand. Therefore, the proposed control strategy ensures efﬁcient operation during all operating conditions without

Description	Parameter	Value

f (Hz)	50

AC microgrid	Vac (V)	415 (l − l)

Pdc,max,TC1 (kW)	10

compromising the inherited ﬂexibility of the droop based scheme. The allocation of the tie-converter’s power for droop 1 and droop 2 control mode depends on the chosen value of L% and H% which are user deﬁnable, and should be tuned to

Tie-converter

Vdc,start,TC1 (V)	402.5

Vdc,nom,TC1 (V)	400.0

Vdc,nom,TC2 (V)	397.5

L%= H%	10%

allow smooth transition between different modes while con- sidering the voltage and power measurement tolerance/errors in the considered microgrid.

With the proposed voltage regulation mode, the overall voltage regulation performance of the DC microgrid can be improved. In particular during the peak load demand, the

The mode transition logic of the tie-converter is given in the logic ﬂow diagram shown in Fig. 2, and the detailed control block diagram of the tie-converter is shown in Fig. 3. Both scenarios have been tested at different load operating conditions to demonstrate the robustness and effectiveness of the proposed scheme.

Stop TCx

No

If Pdc,TCx>L%×Pdc,max,TCx

Yes

Voltage regulation mode (6)

Vdc<Vdc,start,TCx

No

Yes

Start TCx

Operate in droop 1 control mode, (6)

Scenario 1: DC Microgrid with Variable Load

The DC microgrid comprises  microturbine  (Pdc,max,1  = 10 kW), fuel cell (Pdc,max,2 = 5 kW) and variable DC load (PLoad,peak = 20 kW) and it is interlinked with the AC microgrid through a tie-converter (Pdc,max,TC1 = 10 kW), as shown in Fig. 4.

MT: 10 kW

FC: 5 kW

No

If Pdc,TCx> (100-

H)%×Pdc,max,TCx

Yes

Droop 2 control

mode, (6)

Load

Pdc,1

Pdc,2

PLoad

Pdc,TC

1

Tie converter 10 kW

Fig. 2. Logic ﬂow diagram showing mode transitions of tie-converter.

Fig. 4. Scenario 1: DC microgrid with microturbine, fuel cell and load.

AC

microgrid

→	→	→	→

The load in the DC microgrid is varied in steps from 5 kW to  20  kW  (i.e. 5 kW	10 kW	15 kW	20 kW

10 kW). At the 15 kW load demand, the expected loadings    of generator 1 and generator 2 are more than 80%, and the

+−

PI

+

−

PI

Vdc

Idc,TCx

/

Vdc

Proposed control (6)

Pdc,TCx

Vdc,ref,TCx

Idc,ref,TCx

Vd,ref,TCx

voltage of the DC microgrid is below the set threshold of

vac

Vabc,ref,TCx

PWM

iabc,TCx

Vdc Idc,TCx

DQ

to ABC

Vdc,start,TC1

= 402.5 V. This condition will enable the tie-

Qref,TC

Iq,ref,TCx

x

+

−

PI	+

−

PI

QTCx

Iq,TCx

Vdc

/

Vq,ref,TCx

converter  1  to  import  power  from  the  AC   microgrid   and

nominal value of Vdc,nom,TC1 = 400.0 V. This expected regulate the voltage of the DC microgrid at the deﬁned performance can be witnessed from the results shown in Fig. 5.

At the highlight point 1, at 8 s, the voltage of the DC microgrid

Fig. 3. Control block diagram of tie-converter.

decreases below 400 V followed by the step load change from

12

10

Power (kW)

8	Pdc,1

6

4	Pdc,2

2

1

Pdc,TC

0

−2

123	45

0	2	4	6	8	10	12	14	16	18	20

Time (s) (a)

DC Microgrid Voltage (V)

420

410

400

390

Vdc

12 3

45

Voltage regulated at defined nominal value—400 V

380

0	2	4	6	8	10	12	14	16	18	20

Time (s) (b)

Tie-Converter Mode

1

0.5

0

Turn-ON delay

Tie-converter ON/OFF signal

Tie-converter voltage regulation mode ON signal

Tie-converter droop 2 mode ON/OFF signal

123

Droop 1 control

Voltage regulation mode

45

Turn-OFF delay

0	2	4	6	8	10	12	14	16	18	20

Time (s) (c)

Fig. 5.  Scenario 1: Results showing (a) generators and tie-converter power, (b) DC microgrid voltage and (c) tie-converter control signals for four different  load operating conditions.

10 kW to 15 kW.  This voltage drop triggers tie-converter 1   to start in droop 1 control mode at point 2. After starting        in droop 1 control mode, the tie-converter control mode is

×

point 3, since the set threshold (Pdc,TC1 > 10% Pdc,max,TC1 ) immediately transitioned to the  voltage  regulation  mode  at is satisﬁed. At 12 s, the load in the DC microgrid is further

increased from 15 kW to 20 kW, and the power transferred from the AC microgrid is increased accordingly. Throughout the peak-load demand in the DC microgrid from 8 s to 12 s, tie-converter 1 remains operational and regulates the voltage of the DC microgrid. Once the load demand in the DC microgrid is decreased at the highlighted point 4, at 16 s, the tie-converter turns off automatically after a short delay at point 5, as shown in Fig. 5. As demonstrated, tie-converter 1 only operates once all the DC generators are heavily loaded. During its operation, the voltage in the DC microgrid is regulated to the deﬁned nominal value of 400 V. Therefore the proposed strategy has better voltage regulation performance and ensures efﬁcient operation.

  - Scenario 2: DC Microgrid with Non-dispatchable Genera- tor and Load Proﬁle

A non-dispatchable generator–solar  PV  system  is  added to scenario 1, as shown in Fig. 6. The power output of

the solar PV system is based on a continuously varying irradiance proﬁle. The load in scenario 2 also has a varying  proﬁle with a  peak  demand  of  25  kW. This test scenario is developed to further demonstrate the effectiveness of the proposed strategy for various practical operating conditions of renewable generation and load demand.

MT: 10 kW

Pdc,1

FC: 5 kW

P

dc,2

Tie converter

10 kW

Pdc,TC

1

PV: 10 kWp

Pdc,3

Load

PLoad

AC

microgrid

Fig. 6. Scenario 2: DC microgrid with microturbine, fuel cell, solar PV and load.

The load in the DC microgrid increases gradually to the peak value 24.5 kW, and then decreases, as shown in Fig. 7(a).

25	12	3	4

Power (kW)

20

15

10

5

0

0	2	4	6	8	10	12	14	16	18	20

Time (s) (a)

12

10	Pdc,3

Power (kW)

8	Pdc,1

6

4	Pdc,2

2

1

Pdc,TC

0

12	3	4

0	2	4	6	8	10	12	14	16	18	20

Time (s) (b)

DC Microgrid Voltage (V)

420

410

400

390

Vdc

12	3	4

Voltage regulated at defined nominal value—400 V

380

0	2	4	6	8	10	12	14	16	18	20

Time (s) (c)

Tie-Converter Mode

1

0.5

0

Tie-converter ON/OFF signal

Tie-converter voltage regulation mode ON signal

Tie-converter droop 2 mode ON/OFF signal

12

Droop 1 control

Voltage regulation mode

3	4

Droop 1 control

0	2	4	6	8	10	12	14	16	18	20

Time (s)	 (d)

Fig. 7. Scenario 2: Results showing (a) DC microgrid load demand, (b) generators and tie-converter power, (c) DC microgrid voltage and (d) tie-converter control signals at varying solar PV and load operating conditions.

The loading on the DC generators increases with the increasing load demand. At the highlight point 1, the loading on generator

microgrid drops  below  the  set  threshold  of  Vdc,start,TC1  = 1 and generator 2 exceeds 80% and the voltage of the DC

402.5 V when the load demand is very high and the solar

PV output is less. In agreement with the proposed control, tie-converter 1 starts at highlighted point 1 and imports power from the AC microgrid to overcome the power deﬁcit in the DC microgrid while regulating its voltage. Tie-converter 1 operates in the voltage regulating mode from point 2 at 8.5 s to point 3 at 14.2 s. From point 3 and onward, the load in     the DC microgrid deceases such that the tie-converter power

×

output is below 10% Pdc,max,TC1 and this condition requires

the tie-converter to operate in the droop 1 control mode before

it turns off at highlighted point 4 at 16.4 s. From point 4 and onward, the load demand in the DC microgrid is less than   the generation, hence it can be met by the local generators.   As expected, it has been demonstrated that the tie-converter only operates during the  power deﬁcit in the DC microgrid.  In addition, the voltage of the DC microgrid is also regulated by importing power from the AC grid. This behavior depicts the grid-connected mode of the AC microgrid but through a tie-converter.

- CONCLUSION

An autonomous power management scheme has been pre- sented for interlinked AC-DC microgrids having different con- ﬁgurations. The proposed scheme manages the power deﬁcit in the DC microgrid efﬁciently and autonomously. The number of tie-converters in operation has been reduced with the proposed prioritization to avoid unnecessary operational losses. The scheme has demonstrated better voltage regulation in the DC microgrid. The performance and robustness of the proposed scheme have been validated for two different scenarios of the DC microgrid at variable load conditions.

REFERENCES

- J. Rocabert, A. Luna, F. Blaabjerg, and P. Rodr´ıguez, “Control of power converters in AC microgrids,” IEEE Transactions on Power Electronics, vol. 27, no. 11, pp. 4734–4749, Nov. 2012.

- M. Liserre, T. Sauter, and J. Y. Hung, “Future energy systems: inte- grating renewable energy sources into the smart power grid through industrial electronics,” IEEE Industrial Electronics Magazine, vol.4. no. 1, pp. 18–37, Mar. 2010.

- M. Tsili and S. Papathanassiou, “A review of grid code technical requirements for wind farms,” IET Renewable Power Generation, vol.  3, no. 3, pp. 308–332, Sep. 2009.

- T.  Strasser,  F.  Andre´n,  J.  Kathan,  C.  Cecati,  C.  Buccella,  P.  Siano,  P. Leita˜o,  G.  Zhabelova,  V.  Vyatkin,  P.  Vrba,  and  V.  Maˇr´ık,  “A  review of architectures and concepts for intelligence in future electric energy systems,” IEEE Transactions on Industrial Electronics, vol. 62, no. 4, pp. 2424–2438, Apr. 2015.

- A. Kwasinski, “Quantitative evaluation of dc microgrids availability: Effects of system architecture and converter topology design choices,” IEEE Transactions on Power Electronics, vol. 26, no. 3, pp. 835–851, Mar. 2011.

- P. Basak, S. Chowdhury, S. H. N. Dey, S. P. Chowdhury, “A literature review on integration of distributed energy resources in the perspec-  tive of control, protection and stability of microgrid,” Renewable and Sustainable Energy Reviews, vol. 16, no. 8, pp. 5545–5556, Oct. 2012.

- D. E. Olivares, A. Mehrizi-Sani, A. H. Etemadi, C. A. Canizares, R. Iravani, M. Kazerani, A. H. Hajimiragha, O. Gomis-Bellmunt, M. Saeed- ifard, R. alma-Behnke, G. A. Jimenez-Estevez, and N. D. Hatziargyriou, “Trends in microgrid control,” IEEE Transactions on Smart Grid, vol. 5, no. 4, pp. 1905–1919, Jul. 2014.

- N. Hatziargyriou, H. Asano, R. Iravani, and C. Marnay, “Microgrids,” IEEE Power and Energy Magazine, vol. 5, no. 4, pp. 78–94, Jul. /Aug. 2007.

- L. E. Zubieta, “Are microgrids the future of energy?: DC microgrids from concept to demonstration to deployment,” IEEE Electriﬁcation Magazine, vol. 4, no. 2, pp. 37–44, Jun. 2016.

- G. Venkataramanan and C. Marnay, “A larger role for microgrids,” IEEE Power and Energy Magazine, vol. 6, no. 3, pp. 78–82, May-Jun. 2008.

- W. Yuan, J. H. Wang, F. Qiu, C. Chen, C. Q. Kang, and B. Zeng, “Ro- bust optimization-based resilient distribution network planning against natural disasters,” IEEE Transactions on Smart Grid, vol. 7, no. 6, pp. 2817–2826, Nov. 2016.

- N. Nikmehr, S. N. Ravadanegh, “Optimal power dispatch of multi- microgrids at future smart distribution grids,” IEEE Transactions on Smart Grid, vol. 6, no. 4, pp. 1648–1657, Jul. 2015.

- H. Farzin, M. Fotuhi-Firuzabad, M. Moeini-Aghtaie, “Enhancing power system resilience through hierarchical outage management in multi- microgrids,” IEEE Transactions on Smart Grid, vol. 7, no. 6, pp. 2869– 2879, Nov. 2016.

- J. Wu, X. H. Guan, “Coordinated multi-microgrids optimal control al- gorithm for smart distribution management system,” IEEE Transactions on Smart Grid, vol. 4, no. 4, pp. 2174–2181, Dec. 2013.

- N. Hatziargyriou, “Operation of multi-microgrids,” in Microgrids: Ar- chitectures and Control, Wiley-IEEE Press, 2014.

- I. U. Nutkani, P. C. Loh, and F. Blaabjerg, “Distributed operation of interlinked AC microgrids with dynamic active and reactive power tuning,” IEEE Transactions on Industry Applications, vol. 49, no. 5,    pp. 2188–2196, Sep. /Oct. 2013.

- I. U. Nutkani, P. C. Loh, and F. Blaabjerg, “Power ﬂow control of intertied AC microgrids,” IET Power Electronics, vol. 6, no. 7, pp. 1329– 1338, Aug. 2013.

- P. C. Loh, D. Li, Y. K. Chai, and F. Blaabjerg, “Autonomous operation of hybrid microgrid with AC and DC subgrids,” IEEE Transactions on Power Electronics, vol. 28, no. 5, pp. 2214–2223, May 2013.

- P. C. Loh, D. Li, Y. K. Chai, and F. Blaabjerg, “Autonomous control of interlinking converter with energy storage in hybrid AC–DC microgrid,” IEEE Transactions on Industry Applications, vol. 49, no. 3, pp. 1374– 1382, May-Jun. 2013.

- P. C. Loh, D. Li, Y. K. Chai, and F. Blaabjerg, “Autonomous operation of ac-dc microgrids with minimised interlinking energy ﬂow,” IET Power Electronics, vol. 6, no. 8, pp. 1650–1657, Sep. 2013.

- N. Eghtedarpour and E. Farjah, “Power control and management in a hybrid AC/DC microgrid,” IEEE Transactions on Smart Grid, vol. 5, no. 3, pp. 1494–1505, May 2014.

- P. Wang, C. Jin, D. X. Zhu, Y. Tang, P. C. Loh, and F. H. Choo, “Dis- tributed control for autonomous operation of a three-Port AC/DC/DS hybrid microgrid,” IEEE Transactions on Industrial Electronics, vol. 62, no. 2, pp. 1279–1290, Feb. 2015.

- X. Liu, P. Wang, and P. C. Loh, “A hybrid AC/DC microgrid and its coordination control,” IEEE Transactions on Smart Grid, vol. 2, no. 2, pp. 278–286, Jun. 2011.

- M. N. Ambia, A. Al-Durra, and S. M. Muyeen, “Centralized power control strategy for AC-DC hybrid micro-grid system using multi- converter scheme,” in Proceedings of the 37th Annual Conference on IEEE Industrial Electronics Society, 2011, pp. 843–848.

- A. A. A. Radwan and Y. A. R. I. Mohamed, “Bidirectional power man- agement in hybrid AC-DC islanded microgrid system,” in Proceedings of 2014 IEEE PES General Meeting, 2014, pp. 1–5.

- I. U. Nutkani, W. Peng, P. C. Loh, and F. Blaabjerg, “Autonomous economic operation of grid connected DC microgrid,”in Proceedings of the 5th International Symposium on Power Electronics for Distributed Generation Systems (PEDG), 2014, pp. 1–5.

- I. U. Nutkani, W. Peng, P. C. Loh,  and  F.   Blaabjerg,  “Cost-based droop scheme for DC microgrid,” in Proceedings of 2014 IEEE Energy Conversion Congress and Exposition (ECCE), 2014, pp. 765–769.

Inam Ullah Nutkani (SM’14) received a B.E. de- gree in electrical engineering from the NED Uni- versity of Engineering and Technology, Karachi, Pakistan in 2003, and M.S. and Ph.D. degrees in Electrical Power Engineering from the Nanyang Technological University, Singapore, in 2007 and 2014, respectively. He has been working in different domains of power system with power utilities, reg- ulators, industry and research organizations includ- ing K-Electric Karachi, NESCOM Islamabad, West Energy, Singapore, JM Pang & Seah Singapore and

Experimental Power Grid Centre (EPGC), A*STAR Singapore, during 2003 to 2016. Currently he is a Lecturer at the School of Engineering, Royal Melbourne Institute of Technology (RMIT), Melbourne, Australia. His ﬁelds of professional expertise and interest include renewable, distributed generation and microgrids, power electronics and control, and next generation electricity grids.

Poh Chiang Loh Andrew (M’04) received his B.E. degree (Hons) and M.E. degree from the National University of Singapore in 1998 and 2000 respec- tively, and his Ph.D. degree from Monash University, Australia, in 2002, all in electrical engineering. After his studies, he has worked for 10 years as an As- sistant Professor, and a Tenured Associate Professor in the Nanyang Technological University, Singapore. From 2013 to 2016, he was a Tenured Full Profes- sor at Aalborg University, Denmark. Currently, he is a Tenured Full Professor at the Department of

Electronics Engineering, the Chinese University of Hong Kong, China, and a Full Professor at the Beijing Jiaotong University, China. His research interest includes multilevel inverters, converter topological developments, converter control and modulation techniques.

Lasantha Meegahapola (M’11–SM’17) received his B.E. degree in electrical engineering (First Class) from the University of Moratuwa, Sri Lanka in  2006, and his Ph.D. degree from Queen’s University of Belfast, UK in 2010. His doctoral study was based on the investigation of power system stability issues with high wind penetration, and research was conducted in collaboration with EirGrid (Republic of Ireland-TSO). In addition, he conducted exten- sive research studies on coordinated reactive power dispatch during steady-state and dynamic/transient

conditions for networks with high wind penetration. He was a visiting researcher in the Electricity Research Centre, University College Dublin, Ireland (2009/2010). From 2011 to 2014 he was employed as a Lecturer at  the University of Wollongong (UOW) and continues as an Honorary Fellow at UOW. He is currently employed as a Lecturer at Royal Melbourne Institute of Technology (RMIT) Melbourne, Australia. He is a senior member of the IEEE and IEEE Power Engineering Society (PES).

Frede Blaabjerg (M’88–SM’97–F’03) was with ABB-Scandia, Randers, Denmark, from 1987 to 1988. From 1988 to 1992, he was a Ph.D. Student with Aalborg University, Aalborg, Denmark. He became an Assistant Professor in 1992, an Associate Professor in 1996, and a Full Professor of power electronics and drives in 1998. His current research interests include power electronics and its applica- tions such as in wind turbines, PV systems, reli- ability, harmonics and adjustable speed drives. He has received 15 IEEE Prize Paper Awards, the IEEE

PELS Distinguished Service Award in 2009, the EPE-PEMC Council Award in 2010 and the IEEE William E. Newell Power Electronics Award 2014. He was an Editor-in-Chief of the IEEE Transactions on Power Electronics from 2006 to 2012. He has been a Distinguished Lecturer for the IEEE Power Electronics Society from 2005 to 2007 and for the IEEE Industry Applications Society from 2010 to 2011.